# Tutorial (TD) – CIDR Addressing

This tutorial focuses on CIDR (Classless Inter-Domain Routing). Students must use CIDR notation and show all calculation steps (subnet mask, block size, network and broadcast addresses).

## Exercise 1 – CIDR Prefix Interpretation

For each CIDR prefix below, give:
- The subnet mask in dotted-decimal format
- The number of usable hosts

a) /25
b) /26
c) /27
d) /28

## Exercise 2 – CIDR Subnet Analysis

Given the network 192.168.1.0/26:
a) Give the subnet mask.
b) Determine the block size.
c) List all subnet network addresses.
d) Give the broadcast address of the second subnet.

## Exercise 3 – Host Range Calculation

Given the subnet 192.168.10.64/27:
a) Identify the subnet mask.
b) Identify the network address.
c) Identify the broadcast address.
d) Give the usable host range.

## Exercise 4 – CIDR on Class B Network

Given the network 172.16.0.0/20:
a) Give the subnet mask.
b) Determine the block size.
c) List the first four subnet network addresses.

## Exercise 5 – Route Aggregation (Supernetting)

The following networks are advertised separately:
192.168.4.0/24
192.168.5.0/24

a) Can these networks be aggregated?
b) If yes, give the summarized CIDR route.
c) Explain the benefit of route aggregation.

## Exercise 6 – Real-World CIDR Scenario

An organization is assigned the address block 10.0.0.0/22.
a) Give the subnet mask.
b) How many total IP addresses are available?
c) How many usable host addresses are available?
d) Give the network and broadcast addresses.