# Tutorial (TD) – IPv4 Subnetting (Equal-Size Subnets)

This tutorial focuses exclusively on IPv4 subnetting with equal-size subnets.
CIDR notation is NOT allowed. Only dotted-decimal subnet masks must be used.
All subnets created in each exercise must have the same size.
Students must clearly show all calculation steps.

## Exercise 1 – Subnetting a Class C Network

Given the network 192.168.1.0 with the default mask 255.255.255.0:
a) Divide the network into 4 equal subnets.
b) Give the new subnet mask in dotted-decimal format.
c) For each subnet, give:
- Network address
- Broadcast address
- Usable host range

## Exercise 2 – Fixed Number of Subnets

Given the network 192.168.10.0 with mask 255.255.255.0:
a) Create 8 equal-size subnets.
b) Determine the subnet mask.
c) Give the network address of the first 5 subnets.

## Exercise 3 – Host Capacity Verification

The network 192.168.20.0 uses the subnet mask 255.255.255.224.
a) How many equal-size subnets are created from the original Class C network?
b) How many usable hosts are available per subnet?
c) Give the network and broadcast address of the second subnet.

## Exercise 4 – Class B Network Subnetting

Given the network 172.16.0.0 with the default mask 255.255.0.0:
a) Divide the network into 16 equal-size subnets.
b) Give the new subnet mask.
c) Give the network address of the first 4 subnets.

## Exercise 5 – Subnet Increment Method

Given the network 192.168.50.0 and subnet mask 255.255.255.192:
a) Determine the subnet increment.
b) List all subnet network addresses.
c) Give the broadcast address of the last subnet.

## Exercise 6 – Complete Subnet Analysis

Given the network 192.168.100.0 with subnet mask 255.255.255.240:
a) How many equal-size subnets are created?
b) How many usable hosts per subnet?
c) Give the complete details of the third subnet:
- Network address
- Broadcast address
- Usable host range